# Cross Service Instructional Guide

## 1. Selecting the Tree

- Cut a tree approximately 20 feet tall.

- A pine tree works best, ideally one that is standing dead (no new needles). These are already dried internally.

- A fallen tree may also be used, provided it has no termites.

  - Test by striking the trunk firmly with a hammer. If termites emerge or the wood crumbles easily, discard it.

- Avoid using a green (living) tree:

  - Sap makes chopping difficult.

  - The tree will be significantly heavier.

## 2. Preparing the Base and Stage Anchor

- The hole in our stage is 10 inches in diameter, which determines the size of the tree base.

- You will need either:

  - A hole in the stage, or

  - A secure method to anchor against the stage.

- When anchoring against a stage:

  - Build a sturdy 2×4 box for the base to drop into.

  - The tree can weigh 500 pounds or more, so the box must be extremely secure.

  - Screw the box firmly into the stage structure.

## 3. Cutting the Crossbeam

- After cutting the full tree, remove a 7‑foot section from the top.

- This section will become the crossbeam.

## 4. Notching the Main Beam

- Measure the width of the crossbeam.

- Cut notches of that same width into the main (vertical) beam.

- Leave the top 18 inches of the main beam un‑notched.

- This creates a clean, well‑balanced “T” shape.

## 5. Notching the Crossbeam

- Measure the width of the main beam at the notched location.

- Cut corresponding notches into the crossbeam so the pieces interlock.

- Think of it like Lincoln Logs:

  - Notches should be no deeper than 50% of the tree’s thickness.

  - Cutting deeper weakens the structure and reduces load‑bearing strength.

## 6. Pre‑Set Chopping Pattern

- Typically, three notches per piece are used.

- These notches allow for controlled chopping during the sermon.

- The pattern is pre‑set so the pieces naturally fit together when assembled.

## 7. Drying the Wood

- Store all wood in a dry location until the day of the service.

- Wet wood dramatically increases weight and difficulty of handling.

## 8. Fitting the Base

- Before the service, confirm the bottom of the tree fits properly into the base.

- Shape and trim the base as needed for a snug, secure fit.

## 9. Installing Stabilizing Hardware

- Install two heavy‑duty eye screws into the crossbeam.

- These allow ropes to be threaded through to help:

  - Stabilize the cross

  - Control movement as it is raised

## 10. Final Assembly and Securing

- Once the cross is assembled:

  - Secure it with thick rope.

  - Drive a 10‑inch spike through the crossbeam section.

- Before raising the cross, it must be:

  - Fully nailed

  - Fully roped

## 11. Stage Support Framework

- Pre‑assemble a support framework on stage for the cross to rest against while tilting upward.

- Construction:

  - Six cinder blocks stacked together

  - 4×4 beams threaded through the openings

- The structure resembles a sawhorse.

- (See reference video for visual clarification.)

## 12. Raising the Cross

- Use five strong men, in addition to yourself, to raise the cross.

- Choose young, physically capable helpers.

- This structure is extremely heavy, especially with a 10‑inch base tree.

- A song is sung while the cross is raised, which adds powerful spiritual and emotional impact.

## 13. Service Planning and Timing

- Carefully plan what you will say and when you will say it.

- This can be done:

  - With a full script, or

  - With Scripture verses displayed on screens as prompts

- Preparation ensures the moment is both safe and spiritually meaningful.